WAT-G-073

Version 1.0, August 2025

SEPA Guidance: List of groundwater hazardous substances and groundwater standards

Contents

1.	Purpose and scope	2

2.	Groundwater standards for groundwater hazardous substances	2

3.	Groundwater threshold values for selected other substances	63

4.	Supporting notes	80

Disclaimer	82

If you would like this document in an accessible format, such as large print, audio recording or braille, please contact SEPA by emailing equalities@sepa.org.uk

# Purpose and scope

This document sets the list of groundwater hazardous substances that SEPA is required to identify by The Water Environment (River Basin Management Planning: Further Provision) (Scotland) Regulations 2013.

This document also sets out the groundwater standards to be applied when:

- Assessing inputs of substances hazardous to groundwater.

- Assessing risks to future groundwater resources using threshold values.

Further detail on how to use these standards is provided in WAT-G-071, SEPA Guidance: Assigning groundwater assessment criteria for pollutant inputs.

This document presents the numerical standards for selected substances that are commonly assessed for regulatory purposes in Scotland. It is not feasible to provide numerical standards for every possible substance. If the substance you are assessing is not included in this document, you can apply the approach outlined in the Scottish Government Directions 2024 to derive an appropriate standard.

The standards derived from health criteria value (HCV) were based on the information available to SEPA at the time. Applicants or operators may propose alternative values for SEPA’s consideration, provided these are derived in accordance with the World Health Organisation (WHO) methodology and the relevant supporting justification is provided.

For radioactive substances, refer to separate relevant SEPA policy and guidance.

# Groundwater standards for groundwater hazardous substances

Table 1: Groundwater standards for groundwater hazardous substances

| The CAS | Substance Name | Input standard >50m from surface water (µg/L) | Basis for standard | Input standard ≤50m from surface water (µg/L) | Basis for standard | Threshold value (µg/L) | Basis for value | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 93-76-5 | (2,4,5-trichlorophenoxy)acetic acid | 0.05 | Pesticide | 0.05 | Pesticide | 0.075 | Pesticide | - |
| 93-72-1 | (2,4,5-trichlorophenoxy) propionic acid (2,4,5-TP) (Silvex) | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 630-20-6 | 1,1,2,2-tetrachloroethane | 17.4 | HCV-derived | 17.4 | HCV-derived | 26.1 | HCV-derived | - |
| 76-13-1 | 1,1,2-trichlorotrifluoroethane | 90000 | HCV-derived | 90000 | HCV-derived | 135000 | HCV-derived | - |
| 634-66-2 | 1,2,3,4-tetrachlorobenzene | 30 | HCV-derived | 30 | HCV-derived | 45 | HCV-derived | - |
| 634-90-2 | 1,2,3,5-tetrachlorobenzene | 3 | HCV-derived | 3 | HCV-derived | 4.5 | HCV-derived | - |
| 95-94-3 | 1,2,4,5-tetrachlorobenzene | 0.3 | HCV-derived | 0.3 | HCV-derived | 0.45 | HCV-derived | - |
| 120-82-1 | 1,2,4-trichlorobenzene | 35 | DWS | 35 | DWS | 52.5 | DWS | - |
| 95-63-6 | 1,2,4-trimethylbenzene | 30 | HCV-derived | 30 | HCV-derived | 45 | HCV-derived | - |
| 106-93-4 | 1,2-dibromoethane | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 3209-22-1 | 1,2-dichloro-3-nitrobenzene | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 95-50-1 | 1,2-dichlorobenzene | 500 | DWS | 500 | DWS | 750 | DWS | - |
| 78-87-5 | 1,2-dichloropropane | 20 | DWS | 20 | DWS | 30 | DWS | - |
| 96-23-1 | 1,3-dichloro-2-propanol | 0.99 | HCV-derived | 0.99 | HCV-derived | 1.485 | HCV-derived | - |
| 541-73-1 | 1,3-dichlorobenzene | 60 | HCV-derived | 60 | HCV-derived | 90 | HCV-derived | - |
| 542-75-6 | 1,3-dichloropropene | 10 | DWS | 10 | DWS | 15 | DWS | - |
| 89-61-2 | 1,4-dichloro-2-nitrobenzene | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 106-46-7 | 1,4-dichlorobenzene | 150 | DWS | 150 | DWS | 225 | DWS | - |
| 2163-00-0 | 1,6-dichlorohexane | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 106-94-5 | 1-bromopropane | 600 | HCV-derived | 600 | HCV-derived | 900 | HCV-derived | - |
| 544-10-5 | 1-chlorohexane | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 1195-45-5 | 1-fluoro-4-isocyanatobenzene | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 75-99-0 | 2,2-dichloropropanoic acid (Dalapon) | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 576-24-9 | 2,3-dichlorophenol | 9 | HCV-derived | 9 | HCV-derived | 13.5 | HCV-derived | - |
| 78-88-6 | 2,3-dichloropropene | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 95-95-4 | 2,4,5-trichlorophenol | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 108-77-0 | 2,4,6-trichloro-1,3,5-triazine | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 88-06-2 | 2,4,6-trichlorophenol | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | Consider also taste/odour when assessing risks to potable abstractions (see WHO guidance). |
| 120-83-2 | 2,4-dichlorophenol (2,4-DP) | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | Consider also taste/odour when assessing risks to potable abstractions (see WHO guidance). |
| 14861-17-7 | 2,4-dichlorophenoxy-4-aniline (aminofen) | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 94-82-6 | 2,4-dichlorophenoxybutyric acid (2,4-DB) | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 87-65-0 | 2,6-dichlorophenol | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 95-85-2 | 2-amino-4-chlorophenol | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 120-32-1 | 2-benzyl-4-chlorophenol | 0.005 | pesticide | 0.005 | pesticide | 0.0075 | pesticide | - |
| 126-99-8 | 2-chloro-1,3-butadiene | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 95-51-2 | 2-chloroaniline | 10 | HCV-derived | 10 | HCV-derived | 15 | HCV-derived | - |
| 131-09-9 | 2-chloroanthraquinone | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 107-07-3 | 2-chloroethanol | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 95-57-8 | 2-chlorophenol | 150 | DWS | 150 | DWS | 225 | DWS | Consider also taste/odour when assessing risks to potable abstractions (see WHO guidance). |
| 95-49-8 | 2-chlorotoluene | 70 | DWS | 70 | DWS | 105 | DWS | - |
| 95-76-1 | 3,4-dichloroaniline | 0.01 | detection limit | 0.01 | detection limit | 0.01 | detection limit | - |
| 99-54-7 | 3,4-dichloronitrobenzene | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 108-42-9 | 3-chloroaniline | 156 | HCV-derived | 156 | HCV-derived | 234 | HCV-derived | - |
| 108-43-0 | 3-chlorophenol | 9 | HCV-derived | 9 | HCV-derived | 13.5 | HCV-derived | - |
| 107-05-1 | 3-chloropropene | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 100-44-7 | 3-chlorotoluene (alphachlorotoluene) | 1 | DWS | 1 | DWS | 1.5 | DWS | - |
| 55406-53-6 | 3-Iodo-2-propynyl n-butyl carbamate (IPBC) | 600 | HCV-derived | 600 | HCV-derived | 900 | HCV-derived | - |
| 94-81-5 | 4-(4-chloro-2-methylphenoxy) butyric acid | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 1570-64-5 | 4-chloro-2-methylphenol | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 89-63-4 | 4-chloro-2-nitroaniline | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 59-50-7 | 4-chloro-3-methylphenol | 300 | HCV-derived | 300 | HCV-derived | 450 | HCV-derived | - |
| 106-47-8 | 4-chloroaniline | 6 | HCV-derived | 6 | HCV-derived | 9 | HCV-derived | - |
| 106-48-9 | 4-chlorophenol | 9 | HCV-derived | 9 | HCV-derived | 13.5 | HCV-derived | - |
| 106-43-4 | 4-chlorotoluene | 2.5 | DWS | 2.5 | DWS | 3.75 | DWS | - |
| 26172-55-4 | 5-chloro-2-methyl-4-isothiazolin-3-one | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 83-32-9 | acenaphthene | 180 | HCV-derived | 180 | HCV-derived | 270 | HCV-derived | - |
| 79-06-1 | acrylamide | 0.05 | DWS | 0.05 | DWS | 0.075 | DWS | - |
| 15972-60-8 | alachlor | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 309-00-2 | aldrin | 0.015 | DWS | 0.02 | Surface Water ES | 0.0225 | DWS | The pesticides aldrin, dieldrin, endrin & isodrin are assessed together against the surface water ES |
| 85422-92-0; 63449-398 | alkanes, C=>18, chloro | 300 | HCV-derived | 300 | HCV-derived | 450 | HCV-derived | - |
| 85535-84-8 | alkanes, C10-13, chloro | 105 | HCV-derived | 0.8 | Surface Water ES | 157.5 | HCV-derived | - |
| 85535-85-9 | alkanes, C14-17, chloro | 69000 | HCV-derived | 69000 | HCV-derived | 103500 | HCV-derived | - |
| 98-87-3 | alpha,alpha-dichlorotoluene | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 67375-30-8 | alpha-cypermethrin | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 959-98-8 | alpha-endosulfan | 0.05 | pesticide | 0.01 | Surface Water ES | 0.075 | pesticide | - |
| 384-22-5 | alpha-trifluoro-2-nitrotoluene | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 121-17-5 | alpha-trifluoro-3-nitro-4-chloro-toluene | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 98-46-4 | alpha-trifluoro-3-nitrotoluene | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 402-54-0 | alpha-trifluoro-4-nitrotoluene | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 61-82-5 | amitrole | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 101-05-3 | anilazine | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 120-12-7 | anthracene | 0.05 | DWS | 0.05 | DWS | 0.075 | DWS | - |
| n/a | arsenic (inorganic III and V) | 5 | DWS | 5 | DWS | 7.5 | DWS | - |
| 1912-24-9 | atrazine | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 2642-71-9 | azinphos ethyl | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 86-50-0 | azinphos methyl | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 3813-05-6 | benazolin | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 15310-01-7 | benodanil | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 17804-35-2 | benomyl | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 71-43-2 | benzene | 0.5 | DWS | 0.5 | DWS | 0.75 | DWS | - |
| 50-32-8 | benzo(a)pyrene | 0.005 | DWS | 0.00034 | Surface Water ES | 0.0075 | DWS | - |
| 205-99-2 | benzo(b)fluoroanthene | 0.05 | DWS | 0.05 | DWS | 0.075 | DWS | Benzo(a)pyrene is used as a marker compound in surface waters; assessment of generic PAHs should refer to benzo(a)pyrene |
| 191-24-2 | benzo(g,h,i)perylene | 0.05 | DWS | 0.05 | DWS | 0.075 | DWS | Benzo(a)pyrene is used as a marker compound in surface waters; assessment of generic PAHs should refer to benzo(a)pyrene |
| 207-08-9 | benzo(k)fluoranthene | 0.05 | DWS | 0.05 | DWS | 0.075 | DWS | Benzo(a)pyrene is used as a marker compound in surface waters; assessment of generic PAHs should refer to benzo(a)pyrene |
| 42576-02-3 | bifenox | 0.05 | pesticide | 0.024 | Surface Water ES | 0.075 | pesticide | - |
| 82657-04-3 | bifenthrin | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 108-60-1 | bis(2-chloroisopropyl) ether | 100 | DWS | 100 | DWS | 150 | DWS | - |
| n/a - mixture | bitumen | - | - | - | - | - | - | Assess on basis of CAS 8002-05-9, Petroleum oil, based on aliphatic and aromatic hydrocarbon components |
| 314-40-9 | bromacil | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| group | brominated diphenylether (penta-, octa-, deca-) | 0.3 | HCV-derived | 0.3 | HCV-derived | 0.45 | HCV-derived | - |
| 1689-84-5 | bromoxynil (as Bromoxynilphenol) | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 1689-99-2 | bromoxynil octanoate | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 133-06-2 | captan | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 10605-21-7 | carbendazim | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 56-23-5 | carbon tetrachloride (tetrachloromethane) | 1.5 | DWS | 1.5 | DWS | 2.25 | DWS | - |
| 1967-16-4 | chlorbufam | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 57-74-9 | chlordane | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 470-90-6 | chlorfenvinphos | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 1698-60-8 | chloridazon | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 97-00-7 | chloro-2,4-dinitrobenzene | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 88-73-3 | chloro-2-nitrobenzene | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 121-73-3 | chloro-3-nitrobenzene | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 100-00-5 | chloro-4-nitrobenzene | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 79-11-8 | chloroacetic acid | 10 | DWS | 10 | DWS | 15 | DWS | See also threshold value for HAAs as group |
| group | chloroaminotoluene | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | Main substance in group is 4-chloroorth-toluidine, breakdown product of the pesticide chlordimeform. As such this "lead" substance is representative of the parent so follows pesticide value selection |
| 108-90-7 | chlorobenzene | 5 | DWS | 5 | DWS | 7.5 | DWS | - |
| 67-66-3 | chloroform | 50 | DWS | 5 | Surface Water ES | 75 | DWS | - |
| 90-13-1 | chloronaphthalene | 240 | HCV-derived | 240 | HCV-derived | 360 | HCV-derived | - |
| 25567-68-4 | chloronitrotoluene | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 76-06-2 | chloropicrin | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 1897-45-6 | chlorothalonil | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 15545-48-9 | chlorotoluron | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 1982-47-4 | chloroxuron | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 101-21-3 | chlorpropham | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 2921-88-2 | chlorpyrifos | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 1861-32-1 | chlorthal-dimethyl | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 18540-29-9 | chromium VI | 25 | DWS | 6.8 | Surface Water ES | 37.5 | DWS | DWS is for all forms of Cr. |
| 74115-24-5 | clofentezine | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 1702-17-6 | clopyralid | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 56-72-4 | coumaphos | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 26444-49-5 | cresyldiphenyl-phosphate | 60 | HCV-derived | 60 | HCV-derived | 90 | HCV-derived | - |
| 21725-46-2 | cyanazine | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 52315-07-8 | cypermethrin | 0.05 | pesticide | 0.00016 | Surface Water ES | 0.075 | pesticide | - |
| 94361-06-5 | cyproconazole | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 50-29-3 | DDT | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 52918-63-5 | deltamethrin | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 8000-97-3 | demeton | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 17040-19-6 | demeton-S-methyl sulphone | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 333-41-5 | diazinon | 0.05 | pesticide | 0.02 | Surface Water ES | 0.075 | pesticide | - |
| 77-58-7 | dibutyl bis(oxylauroyl)tin | 9 | HCV-derived | 9 | HCV-derived | 13.5 | HCV-derived | - |
| n/a | dibutyltin cation | 0.3 | HCV-derived | 0.3 | HCV-derived | 0.45 | HCV-derived | - |
| 818-08-6 | dibutyltin oxide | 0.3 | HCV-derived | 0.3 | HCV-derived | 0.45 | HCV-derived | - |
| 1194-65-6 | dichlobenil | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 1085-98-9 | dichlofluanid | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 91-94-1 | dichlorobenzidine | 0.4 | DWS | 0.4 | DWS | 0.6 | DWS | - |
| group | dichloronitrobenzene (all isomers) | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 97-23-4 | dichlorophen | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 120-36-5 | dichlorprop | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 62-73-7 | dichlorvos | 0.05 | pesticide | 0.0012 | Surface Water ES | 0.075 | pesticide | - |
| 75736-33-3 | diclobutrazol | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 51338-27-3 | diclofop-methyl | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 99-30-9 | dicloran | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 115-32-2 | dicofol | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 60-57-1 | dieldrin | 0.05 | pesticide | 0.02 | Surface Water ES | 0.075 | pesticide | The pesticides aldrin, dieldrin, endrin & isodrin are assessed together against the surface water ES |
| 2227-17-0 | dienochlor | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 119446-68-3 | difenoconazole | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 35367-38-5 | diflubenzuron | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 83164-33-4 | diflufenican | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 60-51-5 | dimethoate | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 110488-70-5 | dimethomorph | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| group | dioxins | 0.015 | DWS | 0.015 | DWS | 0.0225 | DWS | - |
| 712-48-1 | diphenyl chloroarsine | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 298-04-4 | disulfoton | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 330-54-1 | diuron | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 123-01-3 | dodecyl benzene | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 51450-97-6 | drazoxolon | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 115-29-7 | endosulfan | 0.05 | pesticide | 0.01 | Surface Water ES | 0.075 | pesticide | - |
| 72-20-8 | endrin | 0.015 | DWS | 0.02 | Surface Water ES | 0.0225 | DWS | The pesticides aldrin, dieldrin, endrin & isodrin are assessed together against the surface water ES |
| 106-89-8 | epichlorohydrin | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 135319-73- 2 or 133855-98- 8 | epoxiconazole | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 66230-04-4 | esfenvalerate | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 13194-48-4 | ethoprophos | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 598-14-1 | ethyl dichloroarsine | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 100-41-4 | ethylbenzene | 150 | DWS | 150 | DWS | 225 | DWS | - |
| 2593-15-9 | etridiazole | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 60168-88-9 | fenarimol | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 13356-08-6 | fenbutatin oxide | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 299-84-3 | fenchlorphos | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 122-14-5 | fenitrothion | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 82110-72-3 | fenoxaprop-ethyl | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 71283-80-2 | fenoxaprop-P-ethyl | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 74738-17-3 | fenpiclonil | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 55-38-9 | fenthion | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 900-95-8 | fentin acetate | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 51630-58-1 | fenvalerate | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 52756-22-6 | flamprop-M-isopropyl | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 79622-59-6 | fluazinam | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 70124-77-5 | flucythrinate | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 69770-45-2 | flumethrin | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 206-44-0 | fluoranthene | 37.5 | HCV-derived | 0.0126 | Surface Water ES | 56.25 | HCV-derived | - |
| 77501-90-7 | fluroglycofen-ethyl | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 69377-81-7 | fluroxypyr | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 85509-19-9 | flusilazole | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 76674-21-0 | flutriafol | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 72178-02-0 | fomesafen | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 944-22-9 | fonofos | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 76-44-8 | heptachlor | 0.015 | DWS | 0.0000004 | Surface Water ES | 0.0225 | DWS | - |
| 34783-40-9 | heptenophos | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 25637-99-4 | hexabromocyclododecane (HBCDD) | 4.5 | HCV-derived | 0.0032 | Surface Water ES | 6.75 | HCV-derived | - |
| 118-74-1 | hexachlorobenzene (HCB) | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 87-68-3 | hexachlorobutadiene (HCBD) | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 608-73-1 | hexachlorocyclohexane | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 58-89-9 | hexachlorocyclohexane (lindane) (gamma-HCH) | 0.05 | pesticide | 0.04 | Surface Water ES | 0.075 | pesticide | - |
| 67-72-1 | hexachloroethane | 4.5 | DWS | 4.5 | DWS | 6.75 | DWS | - |
| 1335-87-1 | hexachloronaphthalene | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 3389-71-7 and 2868044-6 | hexachloronorbornadiene | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 79983-71-4 | hexaconazole | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 35554-44-0 | imazalil | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 138261-41-3 | imidacloprid | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 193-39-5 | indeno(1,2,3-cd)pyrene | 0.05 | DWS | 0.05 | DWS | 0.075 | DWS | Benzo(a)pyrene is used as a marker compound in surface waters; assessment of generic PAHs should refer to benzo(a)pyrene |
| 1689-83-4 | ioxynil | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 36734-19-7 | iprodione | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 465-73-6 | isodrin | 0.015 | DWS | 0.02 | Surface Water ES | 0.0225 | DWS | The pesticides aldrin, dieldrin, endrin & isodrin are assessed together against the surface water ES |
| 91465-08-6 | lambda-cyhalothrin | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| n/a | lead (total) | 5 | DWS | 2.4 | Surface Water ES (see comment) | 7.5 | DWS | Surface water ES is worst case bioavailable concentration |
| 330-55-2 | linuron | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 569-64-2 | malachite Green | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 121-75-5 | malathion | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 950-10-7 | mephosfolan | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 21908-53-2 | mercuric oxide | 0.5 | DWS | 0.5 | DWS | 0.75 | DWS | - |
| n/a | mercury compounds (inorganic total) | 0.5 | DWS | 0.5 | DWS | 0.75 | DWS | - |
| 67129-08-2 | metazachlor | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 10265-92-6 | methamidophos | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 950-37-8 | methidathion | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 19937-59-8 | metoxuron | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 7786-34-7 | mevinphos | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 8012-95-1 | mineral oil | - | - | - | - | - | - | Assess on basis of CAS 8002-05-9, Petroleum oil, based on aliphatic and aromatic hydrocarbon components |
| 1746-81-2 | monolinuron | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 88671-89-0 | myclobutanil | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 33855-47-9 | N-(4-bromophenyl) methyl-1,2-ethanediamine | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 63284-71-9 | nuarimol | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 58810-48-3 | ofurace | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 1113-02-6 | omethoate | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 19666-30-9 | oxadiazon | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 301-12-2 | oxydemeton-methyl | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 76738-62-0 | paclobutrazol | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 56-38-2 | parathion | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 298-00-0 | parathion-methyl | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 66246-88-6 | penconazole | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 608-93-5 | pentachlorobenzene | 2 | HCV-derived | 0.014 | Surface Water ES | 3 | HCV-derived | - |
| 76-01-7 | pentachloroethane | 0.01 | detection limit | 0.01 | detection limit | 0.01 | detection limit | - |
| 87-86-5 | pentachlorophenol (PCP) | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 2307-68-8 | pentanochlor | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 1763-23-1 | perfluorooctane sulfonic acid and its salts (PFOS) | 0.05 | DWS | 0.0013 | Surface Water ES | 0.075 | DWS | See also potable standard for Sum of PFAS |
| 52645-53-1 | permethrin | 0.05 | pesticide | 0.002 | Surface Water ES | 0.075 | pesticide | - |
| 8002-05-9 | petroleum oil TPH Aliphatic EC 5-6 | 7500 | DWS | 7500 | DWS | 11250 | DWS | Consider also taste and odour when assessing risks to potable abstractions |
| 8002-05-9 | petroleum oil TPH Aliphatic EC >6-8 | 2700 | Solubility criteria | 2700 | Solubility criteria | 4050 | Solubility criteria | May be modified to account for effective solubility if present in mixture of fractions Consider also taste and odour when assessing risks to potable abstractions |
| 8002-05-9 | petroleum oil TPH Aliphatic EC >8-10 | 150 | DWS | 150 | DWS | 225 | DWS | May be modified to account for effective solubility if present in mixture of fractions Consider also taste and odour when assessing risks to potable abstractions |
| 8002-05-9 | petroleum oil TPH Aliphatic EC >8-10 | 150 | DWS | 150 | DWS | 225 | DWS | May be modified to account for effective solubility if present in mixture of fractions Consider also taste and odour when assessing risks to potable abstractions |
| 8002-05-9 | petroleum oil TPH Aliphatic EC >10-12 | 17 | Solubility criteria | 17 | Solubility criteria | 25.5 | Solubility criteria | May be modified to account for effective solubility if present in mixture of fractions Consider also taste and odour when assessing risks to potable abstractions |
| 8002-05-9 | petroleum oil TPH Aliphatic EC >12-16 | 0.38 | Solubility criteria | 0.38 | Solubility criteria | 0.57 | Solubility criteria | May be modified to account for effective solubility if present in mixture of fractions Consider also taste and odour when assessing risks to potable abstractions |
| 8002-05-9 | petroleum oil TPH Aliphatic EC >16-35 | 0.0015 | Solubility criteria | 0.0015 | Solubility criteria | 0.00225 | Solubility criteria | May be modified to account for effective solubility if present in mixture of fractions Consider also taste and odour when assessing risks to potable abstractions |
| 8002-05-9 | petroleum oil TPH Aromatic EC 5-7 | n/a | n/a | n/a | n/a | See comment |  | Refer to CAS 71-43-2 benzene |
| 8002-05-9 | petroleum oil TPH Aromatic EC >7-8 | n/a | n/a | n/a | n/a | See comment |  | Refer to CAS 108-88-3 toluene |
| 8002-05-9 | petroleum oil TPH Aromatic EC >8-10 | n/a | n/a | n/a | n/a | See comment |  | Refer to CAS 100-41-4 ethylbenzene |
| 8002-05-9 | petroleum oil TPH Aromatic EC >10-12 | 45 | DWS | 45 | DWS | 67.5 | DWS | Consider also taste and odour when assessing risks to potable abstractions |
| 8002-05-9 | petroleum oil TPH Aromatic EC >12-16 | 45 | DWS | 45 | DWS | 67.5 | DWS | Consider also taste and odour when assessing risks to potable abstractions |
| 8002-05-9 | petroleum oil TPH Aromatic EC >16-21 | 45 | DWS | 45 | DWS | 67.5 | DWS | Consider also taste and odour when assessing risks to potable abstractions |
| 8002-05-9 | petroleum oil TPH Aromatic EC >21-35 | 3.3 | Solubility criteria | 3.3 | Solubility criteria | 4.95 | Solubility criteria | May be modified to account for effective solubility if present in mixture of fractions Consider also taste and odour when assessing risks to potable abstractions |
| 62-38-4 | phenylmercury acetate | 0.15 | HCV-derived | 0.15 | HCV-derived | 0.225 | HCV-derived | - |
| 298-02-2 | phorate | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 2310-17-0 | phosalone | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 29232-93-7 | pirimiphos-methyl | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 1336-36-3 (category) | polychlorinated biphenyls (PCBs) | n/a | n/a | n/a | n/a | See comment |  | Refer to dioxins to cover risk from dioxins & dioxine-like PCBs |
| 67747-09-5 | prochloraz | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 1918-16-7 | propachlor | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 709-98-8 | propanil | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 111479-05-1 | propaquizafop | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 31218-83-4 | propetamphos | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 60207-90-1 | propiconazole | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 23950-58-5 | propyzamide | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 94125-34-5 | prosulfuron | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 13457-18-6 | pyrazophos | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 55512-33-9 | pyridate | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 88283-41-4 | pyrifenox | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 13593-03-8 | quinalphos | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 82-68-8 | quintozene | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 76578-14-8 | quizalofop-ethyl | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 122-34-9 | simazine | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 100-42-5 | styrene | 10 | DWS | 10 | DWS | 15 | DWS | - |
| 8001-58-9 | tar oil | See comment |  | See comment |  | See comment |  | Assess on basis of CAS 8002-05-9, Petroleum oil, based on aliphatic and aromatic hydrocarbon components |
| 107534-96-3 | tebuconazole | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 83121-18-0 | teflubenzuron | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 79538-32-2 | tefluthrin | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 5902-51-2 | terbacil | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 5915-41-3 | terbuthylazine | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 79-27-6 | tetrabromoethane | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 1461-25-2 | tetrabutyltin | 0.05 | detection limit | 0.05 | detection limit | 0.05 | detection limit | - |
| 12408-10-5 | tetrachlorobenzene (all isomers) | 0.3 | HCV-derived | 0.3 | HCV-derived | See comment |  | Assess on basis of CAS 95-94-3, 1,2,4,5-tetrachlorobenzene, if analysis does not identify substituent patterns of components |
| 22248-79-9 | tetrachlorvinphos | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 116-29-0 | tetradifon | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 640-15-3 | thiometon | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 23564-05-8 | thiophanate-methyl | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 137-26-8 | thiram | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 57018-04-9 | tolclofos-methyl | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 108-88-3 | toluene | 350 | DWS | 148 | Surface Water ES | 933 | DWS | Consider also taste/odour when assessing risks to potable abstractions (see WHO guidance) |
| 43121-43-3 | triadimefon | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 55219-65-3 | triadimenol | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 2303-17-5 | triallate | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 82097-50-5 | triasulfuron | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 24017-47-8 | triazophos | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 72459-58-6 | triazoxide | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 126-73-8 | tributyl-phosphate | 240 | HCV-derived | 240 | HCV-derived | 360 | HCV-derived | - |
| 56-35-9 | tributyltin oxide (TBTO) | 1 | DWS | 0.0004 | Surface Water ES | 1.5 | DWS | - |
| 52-68-6 | trichlorfon | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 76-03-9 | trichloroacetic acid | 30 | DWS | 30 | DWS | 45 | DWS | See also threshold value for HAAs as group |
| 75-87-6 | trichloroethanal | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 25323-89-1 | trichloroethane | 2.5 | DWS | 2.5 | DWS | 3.75 | DWS | Assess on basis of CAS 79-00-5: 1,1,2-trichloroethane |
| 79-01-6 | trichloroethylene | 5 | DWS | 5 | DWS | 7.5 | DWS | - |
| 25167-82-2 | trichlorophenol (all isomers) | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | Two out of three isomers have majority pesticide use |
| 55335-06-3 | triclopyr | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 1330-78-5 | tricresyl-phosphate | 105 | HCV-derived | 105 | HCV-derived | 157.5 | HCV-derived | - |
| 1912-26-1 | trietazine | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 1582-09-8 | trifluralin | 0.05 | pesticide | 0.06 | Surface Water ES | 0.075 | pesticide | - |
| 26644-46-2 | triforine | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 25103-12-2 or 78-42-2 | trioctyl-phosphate | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | HCV not reliable |
| 115-86-6 | triphenyl-phosphate | 350 | HCV-derived | 350 | HCV-derived | 525 | HCV-derived | - |
| 1262-21-1 | triphenyltin oxide (TPTO) | 1 | HCV-derived | 1 | HCV-derived | 1.5 | HCV-derived | - |
| 126-72-7 | tris(2,3-bromo-1-propyl)phosphate | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 25155-23-1 | trixylenyl-phosphate | 0.1 | detection limit | 0.1 | detection limit | 0.1 | detection limit | - |
| 2275-23-2 | vamidothion | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 50471-44-8 | vinclozolin | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |
| 75-01-4 | vinyl chloride | 0.25 | DWS | 0.25 | DWS | 0.375 | DWS | - |
| 1330-20-7 | xylene | 250 | DWS | 60 | Surface Water ES | 375 | DWS | Consider also taste/odour when assessing risks to potable abstractions (see WHO guidance). |
| 137-30-4 | ziram | 0.05 | pesticide | 0.05 | pesticide | 0.075 | pesticide | - |

# Groundwater threshold values for selected other substances

Table 2: Groundwater threshold values for selected other substances

| CAS Reference | Substance Name | Threshold value (µg/L) | Basis for value | Comment |
| --- | --- | --- | --- | --- |
| 120-23-0 | (2-naphthyloxy)acetic acid | 0.075 | Pesticide |  |
| 71-55-6 | 1,1,1-trichloroethane | 1500 | DWS |  |
| 79-00-5 | 1,1,2-trichloroethane | 3.75 | DWS |  |
| 75-34-3 | 1,1-dichloroethane | 3.75 | DWS |  |
| 75-35-4 | 1,1-dichloroethene | 105 | DWS |  |
| 107-06-2 | 1,2-dichloroethane | 2.25 | DWS |  |
| 540-59-0 | 1,2-dichloroethene | 37.5 | DWS |  |
| 86-87-3 | 1-naphthylacetic acid | 0.075 | pesticide | Used to promote growth but covered by plant protection products regulation |
| 133-32-4 | 4-indol-3-butyric acid | 0.075 | detection limit |  |
| 116-06-3 | aldicarb | 0.075 | pesticide |  |
| 593-81-7 | alkylaryl trimethylammonium chloride | 0.075 | detection limit | CAS refers to trimethylamine hydrochloride; alkylaryl substituent is non-specific therefore healthbased standards assessment not possible |
| group | alkyl benzalkonium chloride | 0.075 | detection limit | Group of substances with no definition of chain length, therefore health-based standards assessment not possible |
| 55635-13-7 | alloxydim-sodium | 0.075 | pesticide |  |
| n/a | aluminium | 150 | DWS | Not assessed by JAGDAG |
| 7784-26-1 | aluminium ammonium sulphate | 0.075 | pesticide |  |
| 10043-01-3 | aluminium sulphate | 0.075 | pesticide |  |
| 120923-37-7 | amidosulfuron | 0.075 | pesticide |  |
| 33089-61-1 | amitraz | 0.075 | pesticide |  |
| 7664-41-7 | ammonia | 375 | DWS | For ammonium ions and ammonia. Whilst odour thresholds for ammonia have been documented, these values are higher than the UK DWS for ammonia and ammonium ions combined. Based on the interconversion of the ion and ammonia depending on water chemistry, we recommend using the UK DWS standard. |
| 506-87-6 | ammonium carbonate | 375 | DWS | As per ammonium ions and ammonia |
| 84-65-1 | anthraquinone | 0.075 | pesticide |  |
| 7440-36-0 and 1309-64-4 | antimony (V and III) | 3.75 | DWS |  |
| 3337-71-7 | asulam | 0.075 | pesticide |  |
| 4658-28-0 | aziprotryne | 0.075 | pesticide |  |
| n/a | barium | 975 | DWS | Not determined by JAGDAG |
| 71626-11-4 | benalaxyl | 0.075 | pesticide |  |
| 22781-23-3 | bendiocarb | 0.075 | pesticide |  |
| 82560-54-1 | benfuracarb | 0.075 | pesticide |  |
| 25057-89-0 | bentazone | 0.075 | pesticide |  |
| n/a | beryllium | 9 | DWS | Not determined by JAGDAG. Not a formal recommendation by WHO, since beryllium is rarely found in drinking water |
| 80-05-7 | bisphenol A | 1.875 | DWS | Not determined by JAGDAG |
| 55179-31-2 | bitertanol | 0.075 | pesticide |  |
| n/a | boron (as boron III) | 750 | DWS |  |
| 15541-45-4 | bromate | 7.5 | DWS |  |
| 41483-43-6 | bupirimate | 0.075 | pesticide |  |
| 7440-43-9 | cadmium | 3.75 | DWS |  |
| n/a | calcium | See comment |  | There is no standard for calcium because there is no UK or WHO potable standard plus it does not routinely cause pollution by itself and is normally found in combination with other, more problematic, pollutants. |
| 63-25-2 | carbaryl | 0.075 | pesticide |  |
| 16118-49-3 | carbetamide | 0.075 | pesticide |  |
| 1563-66-2 | carbofuran | 0.075 | pesticide |  |
| 75-15-0 | carbon disulfide | 450 | HCV-derived |  |
| 55285-14-8 | carbosulfan | 0.075 | pesticide |  |
| 5234-68-4 | carboxin | 0.075 | pesticide |  |
| 57-09-0 | cetrimide | 0.075 | detection limit |  |
| 14866-68-3 | chlorate | 0.1875 | DWS | Not determined by JAGDAG |
| n/a | chloride | 187500 | DWS | Not determined by JAGDAG |
| 7782-50-5 | chlorine | 3750 | DWS |  |
| 14998-27-7 | chlorite | 0.1875 | DWS | Not determined by JAGDAG |
| 16065-83-1 | chromium III | 37.5 | DWS | Based on DWS for total Cr |
| n/a | cobalt (based on cobalt II using carbonate & sulphate salt data) | 103.5 | HCV-derived |  |
| group | copper (soluble compounds) | 1500 | DWS |  |
| 95-48-7 | cresylic acid (o-, m- and p-cresol) | 225 | HCV-derived |  |
| 11096-18-9 | cufraneb | 0.075 | pesticide |  |
| 33113-08-5 | cupric ammonium carbonate | 0.075 | pesticide |  |
| 57-12-5 | cyanide | 37.5 | DWS | Consider also taste/odour when assessing risks to potable abstractions (see WHO guidance) |
| 101205-02-1 | cycloxydim | 0.075 | pesticide |  |
| 57966-95-7 | cymoxanil | 0.075 | pesticide |  |
| 1596-84-5 | daminozide | 0.075 | pesticide |  |
| 533-74-4 | dazomet | 0.075 | pesticide |  |
| 13684-56-5 | desmedipham | 0.075 | pesticide |  |
| 1014-69-3 | desmetryn | 0.075 | pesticide |  |
| 117-81-7 | di(2-ethylhexyl) phthalate | 6 | DWS |  |
| 84-74-2 | dibutyl phthalate | 15 | DWS |  |
| 75-09-2 | dichloromethane | 15 | DWS |  |
| 49866-87-7 | difenzoquat | 0.075 | pesticide |  |
| 18467-77-1 | dikegulac | 0.075 | pesticide |  |
| 39300-45-3 | dinocap | 0.075 | pesticide |  |
| 957-51-7 | diphenamid | 0.075 | pesticide |  |
| 122-39-4 | diphenyl amine | 0.075 | pesticide |  |
| 2764-72-9 | diquat | 0.075 | pesticide |  |
| 3347-22-6 | dithianon | 0.075 | pesticide |  |
| 1593-77-7 | dodemorph | 0.075 | pesticide |  |
| 2439-10-3 | dodine | 0.075 | pesticide |  |
| 23947-60-6 | ethirimol | 0.075 | pesticide |  |
| 26225-79-6 | ethofumesate | 0.075 | pesticide |  |
| 107-21-1 | ethylene glycol | 225 | HCV-derived |  |
| 39515-41-8 | fenpropathrin | 0.075 | pesticide |  |
| 67306-00-7 | fenpropidin | 0.075 | pesticide |  |
| 67306-03-0 | fenpropimorph | 0.075 | pesticide |  |
| 101-42-8 | fenuron | 0.075 | pesticide |  |
| 14484-64-1 | ferbam | 0.075 | pesticide |  |
| 142459-58-3 | flufenacet | 0.075 | pesticide |  |
| 7681-49-4 | fluoride | 1125 | DWS | Not determined by JAGDAG |
| 50-00-0 | formaldehyde | 1950 | DWS |  |
| 39148-24-8 | fosetyl-aluminium | 0.075 | pesticide |  |
| 3878-19-1 | fuberidazole | 0.075 | pesticide |  |
| 57646-30-7 | furalaxyl | 0.075 | pesticide |  |
| 77-06-5 | gibberellic acid | 0.075 | pesticide | Naturally occurring plant growth hormone but covered by EU plant protection products regulation |
| group | gibberellins | 0.075 | pesticide | Naturally occurring plant growth hormone but covered by EU plant protection products regulation |
| 111-30-8 | gluteraldehyde | 450 | HCV-derived |  |
| 108173-90-6 | guazatine | 0.075 | pesticide |  |
| group | halo acetic acids (HAA) | 45 | DWS | Assessed as summed total of monochloro-, dichloro-, trichloro-acetic acid and mono- and dibromo-acetic acid. For 79-11-8 chloroacetic acid and 76-03-9 trichloroacetic acid see also relevant hazardous input standards. |
| 1024-57-3 | heptachlor epoxide | 0.0225 | DWS |  |
| 51235-04-2 | hexazinone | 0.075 | pesticide |  |
| 10004-44-1 | hymexazol | 0.075 | pesticide |  |
| 81405-85-8 | imazamethabenz-methyl | 0.075 | pesticide |  |
| 81335-37-7 | imazaquin | 0.075 | pesticide |  |
| 7553-56-2 | iodine | 76.5 | HCV-derived |  |
| n/a | iron | 150 | DWS | Not determined by JAGDAG |
| 34123-59-6 | isoproturon | 0.075 | pesticide |  |
| 82558-50-7 | isoxaben | 0.075 | pesticide |  |
| 2164-08-1 | lenacil | 0.075 | pesticide |  |
| n/a | magnesium | See comment |  | Not determined by JAGDAG. There is no standard for magnesium because there is no UK or WHO potable standard plus it does not routinely cause pollution by itself and is normally found in combination with other, more problematic, pollutants. |
| 123-33-1 | maleic hydrazide | 0.075 | pesticide |  |
| 8018-01-7 | mancozeb | 0.075 | pesticide |  |
| 12427-38-2 | maneb | 0.075 | pesticide |  |
| n/a | manganese | 37.5 | DWS | Not determined by JAGDAG |
| 7085-19-0 | mecoprop | 0.03 | DWS |  |
| 24307-26-4 | mepiquat | 0.075 | pesticide |  |
| 57837-19-1 | metalaxyl | 0.075 | pesticide |  |
| 9002-91-9 | metaldehyde | 0.075 | pesticide |  |
| 41394-05-2 | metamitron | 0.075 | pesticide |  |
| 137-42-8 | metam-sodium | 0.075 | pesticide |  |
| 18691-97-9 | methabenzthiazuron | 0.075 | pesticide |  |
| 2032-65-7 | methiocarb | 0.075 | pesticide |  |
| 1634-04-4 | methyl tertiary butyl ether (MTBE) | 1350 | HCV-derived | Consider also taste/odour when assessing risks to potable abstractions (see WHO guidance) |
| 21087-64-9 | metribuzin | 0.075 | pesticide |  |
| 74223-64-6 | metsulfuron-methyl | 0.075 | pesticide |  |
| n/a | molybdenum | 52.5 | DWS |  |
| 142-59-6 | nabam | 0.075 | pesticide |  |
| 91-20-3 | naphthalene | 90 | HCV-derived |  |
| 15299-99-7 | napropamide | 0.075 | pesticide |  |
| n/a | nickel (as nickel II) | 15 | DWS |  |
| 54-11-5 | nicotine | 3.6 | HCV-derived |  |
| 14797-55-8 | nitrate | 37500 | DWS | Not determined by JAGDAG |
| 14797-65-0 | nitrites | 375 | DWS | Not determined by JAGDAG |
| 10552-74-6 | nitrothal-isopropyl | 0.075 | pesticide |  |
| 25154-52-3 | Nonylphenol | 0.3 | SW ES | Not determined by JAGDAG |
| 26530-20-1 | octhilinone | 0.075 | pesticide |  |
| 19044-88-3 | oryzalin | 0.075 | pesticide |  |
| 77732-09-3 | oxadixyl | 0.075 | pesticide |  |
| 23135-22-0 | oxamyl | 0.075 | pesticide |  |
| 10380-28-6 | oxine-copper | 0.075 | pesticide |  |
| 5259-88-1 | oxycarboxin | 0.075 | pesticide |  |
| 4685-14-7 | paraquat | 0.075 | pesticide |  |
| 40487-42-1 | pendimethalin | 0.075 | pesticide |  |
| group | perfluoralkyl and polyfluoralkyl substances (PFAS) | 0.075 | DWS | The potable standard is assessed as the sum of 20 PFAS For PFOS see relevant hazardous substance input standard. |
| 79-21-0 | peroxyacetic acid | 0.075 | pesticide |  |
| 13684-63-4 | phenmedipham | 0.075 | pesticide |  |
| 108-95-2 | phenol | 2250 | HCV-derived |  |
| 7664-38-2 | phosphoric acid | See comment |  | There is no standard for phosphoric acid, or phosphate, because there is no UK or WHO potable standard plus it does not routinely cause groundwater pollution by itself and is normally found in combination with other, more problematic, pollutants. |
| 23103-98-2 | pirimicarb | 0.075 | pesticide |  |
| 9003-05-8 | polyacrylamide (anionic) | 0.075 | detection limit | No data to assign a health derived value |
| 7440-09-7 | potassium | See comment |  | Not determined by JAGDAG. There is no standard for potassium because there is no UK or WHO potable standard plus it does not routinely cause pollution by itself and is normally found in combination with other, more problematic, pollutants. |
| 7287-19-6 | prometryn | 0.075 | pesticide |  |
| 25606-41-1 | propamocarb hydrochloride | 0.075 | pesticide |  |
| 122-42-9 | propham | 0.075 | pesticide |  |
| 114-26-1 | propoxur | 0.075 | pesticide |  |
| 57-55-6 | propylene glycol | 5250 | DWS |  |
| 8003-34-7 | pyrethrins | 0.075 | pesticide |  |
| 2439-01-2 | quinomethionate | 0.075 | pesticide |  |
| 10453-86-8 | resmethrin | 0.075 | pesticide |  |
| 83-79-4 | rotenone | 0.075 | pesticide |  |
| 7782-49-2 | selenium | 7.5 | DWS |  |
| 74051-80-2 | sethoxydim | 0.075 | pesticide |  |
| n/a | sodium (as sodium I) | 150000 | DWS | Not determined by JAGDAG |
| 7775-09-9 | sodium chlorate | 0.075 | pesticide |  |
| 10022-70-5 and 768152-9 | sodium hypochlorite | 0.075 | pesticide |  |
| 3926-62-3 | sodium monochloroacetate | 0.075 | pesticide |  |
| 1330-43-4 | sodium tetraborate | 0.075 | pesticide |  |
| n/a | sulphate | 187500 | DWS | Not determined by JAGDAG |
| 35256-85-0 | tebutam | 0.075 | pesticide |  |
| 886-50-0 | terbutryn | 0.075 | pesticide |  |
| 127-18-4 | tetrachloroethylene | 7.5 | DWS |  |
| n/a | thallium (as thallium I) | 1.5 | DWS |  |
| 148-79-8 | thiabendazole | 0.075 | pesticide |  |
| 79277-27-3 | thifensulfuron-methyl | 0.075 | pesticide |  |
| 59669-26-0 | thiodicarb | 0.075 | pesticide |  |
| 87820-88-0 | tralkoxydim | 0.075 | pesticide |  |
| 101200-48-0 | tribenuron-methyl | 0.075 | pesticide |  |
| 12002-48-1 | trichlorobenzene | See comment |  | Assess on basis of CAS 120-82-1: 1,2,4-trichlorobenzene |
| group | trihalomethanes | 75 | DWS | Assessed as the summed total of chloroform, bromoform, dibromochloromethane, and bromodichloromethane. For 67-66-3 chloroform see also relevant hazardous substance input standard |
| n/a | uranium | 22.5 | DWS | Not determined by JAGDAG |
| n/a | vanadium | 11.25 | detection limit | Not determined by JAGDAG |
| 7440-66-6 | zinc | 3750 | DWS | Consider also taste/odour when assessing risks to potable abstractions (see WHO guidance). |
| 12122-67-7 | zineb | 0.075 | pesticide |  |
| 9006-42-2 | zineb-ethylenethiuram disulphide | 0.075 | pesticide |  |

# Supporting notes

For radioactive substances, the groundwater standards are dose based and assessed by the risk to members of the public and non-human organisms and their habitats. Refer to the relevant SEPA policy and guidance on assessing radioactive substances.

For metals and cationic or anionic substances, in many cases no CAS number is given because a CAS number does not exist for the substance as assessed (ie the dissolved ion).

"DWS" means the standard was derived based on a drinking water standard or guideline set by the Scottish Government, the World Health Organisation or if not available another international toxicology-based or national drinking water value. Where the current Scottish DWS for public and private water supplies do not align, SEPA have adopted the lower value.

"HCV-derived" means the standard was derived based on a peer reviewed Regulatory Health Criteria value that was used to calculate a water concentration threshold according to WHO methodology.

"MRV derived" means the standard was derived based on minimum reporting values, which in turn are based on limits of quantification

For PAHs there is only an annual average EQS for benzo(a)pyrene as this substance is used as a marker for PAHs, so when assessing close to surface waters this substance should be used to indicate risk.

"pesticide" means a value based on the potable limits for pesticides set out Scottish potable standards. As well as a potable standard for individual pesticides, note the potable standards also include a Total Pesticides limit of 0.5µg/l. Both the individual and cumulative risks should be considered. Note also for Aldrin, Dieldrin, Endrin & Isodrin that the surface water Environmental Standard is based on summed concentrations which means that analysis of all four substances is required to judge compliance.

“Sum of PFAS” is as set out in the Scottish potable standards and comprises the arithmetic sum of perfluorobutanoic acid, perfluoropentanoic acid, perfluorohexanoic acid, perfluoroheptanoic acid, perfluorooctanoic acid, perfluorononanoic acid, perfluorodecanoic acid, perfluoroundecanoic acid, perfluorododecanoic acid, perfluorotridecanoic acid, perfluorobutane sulfonic acid, perfluoropentane sulfonic acid, perfluorohexane sulfonic acid, perfluoroheptane sulfonic acid, perfluorooctane sulfonic acid, perfluorononane sulfonic acid, perfluorodecane sulfonic acid, perfluoroundecane sulfonic acid, perfluorododecane sulfonic acid, and perfluorotridecane sulfonic acid. There is no current potable standard for other PFAS substances.

"Taste/odour criteria" means the threshold concentration at which the substance has detrimental effects for taste and odour in water is lower than the toxicologically-based value.

Although a recent hazardous determination has been made for CrVI, the standard is still based on total chromium (dissolved). This is because to date no accepted standard for CrVI has been established, unlike total chromium for which a value is available under (ii) above, and because of practical issues with speciated analysis. When comparing laboratory data with the assessment criteria, SEPA recommend that dissolved or filtered chromium data is assumed to be CrVI unless proven otherwise. If no dissolved or filtered data available, SEPA recommend professional judgement be applied as to likely speciation based on the hydrogeological setting.

For the input standards for substances hazardous to groundwater, the 50m distance is measured from the downgradient edge of the source. If the source area straddles the 50m threshold, then either:

- Zone the source and assess each section separately; or

- Alternatively apply the lower of the input standards across the whole source.

The latter approach is simpler but more conservative.

The values quoted in Table 1 for input standards for substances hazardous to groundwater for sources within 50m of surface waters are aligned with the freshwater environmental standards. If the closest surface waters are transitional or coastal waters, then the relevant environmental standards for transitional or coastal waters may be taken into consideration.

# Disclaimer

Whilst every effort has been made to ensure the accuracy of this guidance, SEPA gives no warranty, covenant or undertaking (express or implied) regarding the fitness for purpose of, or any error, omission or discrepancy in this guidance. Reliance on its contents and the contents of any websites that are linked to or from this guidance is entirely at the user’s own risk. SEPA is not liable for any loss or damage that may come from using this guidance. This includes:

- any direct, indirect and consequential losses

- any loss or damage caused by civil wrongs, breach of contract or otherwise

SEPA reserves the right to depart from this guidance and take appropriate action as it considers necessary or appropriate. Operators are responsible for ensuring that they are compliant with the law. If necessary, independent legal / specialist advice should be sought.